# Hospitality Profile for HDMI CEC v1.3

Version 1.10

Up to V0.90 to Forum Review; V.91 to 0.99 to incorporate post Forum revisions

V1.0 for released specifications

2 March 2012

Editorial instructions are provided in red italic type.

These instructions do not print, but should be removed from the final document.

About HTNG

Hotel Technology Next Generation (HTNG) is a non-profit association with a mission to foster, through collaboration and partnership, the development of next-generation systems and solutions that will enable hoteliers and their technology vendors to do business globally in the 21st century; to be recognized as a leading voice of the global hotel community, articulating the technology requirements of hotel companies of all sizes to the vendor community; and to facilitate the development of technology models for hospitality that will foster innovation, improve the guest experience, increase the effectiveness and efficiency of hotels, and create a healthy ecosystem of technology suppliers.

Copyright 2012, Hotel Technology Next Generation

All rights reserved.

No part of this publication may be reproduced, stored in a retrieval system, or transmitted, in any form or by any means, electronic, mechanical, photocopying, recording, or otherwise, without the prior permission of the copyright owner.

For any software code contained within this specification, permission is hereby granted, free-of-charge, to any person obtaining a copy of this specification (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the above copyright notice and this permission notice being included in all copies or substantial portions of the Software.

Manufacturers and software providers shall not claim compliance with portions of the requirements of any HTNG specification or standard, and shall not use the HTNG name or the name of the specification or standard in any statements about their respective product(s) unless the product(s) is (are) certified as compliant to the specification or standard.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, AND NON-INFRINGEMENT.  IN NO EVENT SHALL THE AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES, OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF, OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

Permission is granted for implementers to use the names, labels, etc. contained within the specification.  The intent of publication of the specification is to encourage implementations of the specification.

This specification has not been verified for avoidance of possible third-party proprietary rights.  In implementing this specification, usual procedures to ensure the respect of possible third-party intellectual property rights should be followed.

The names Hotel Technology Next Generation and HTNG, and logos depicting these names, are trademarks of Hotel Technology Next Generation.  Permission is granted for implementers to use the aforementioned names in technical documentation for the purpose of acknowledging the copyright and including the notice required above.  All other use of the aforementioned names and logos requires the permission of Hotel Technology Next Generation, either in written form or as explicitly permitted for the organization’s members through the current terms and conditions of membership.

Table of Contents

1	This Specification at a Glance	4

2	Document Information	6

2.1	Document History	6

2.2	Document Purpose	7

2.3	Scope	7

2.4	Relationship to Other Standards	7

2.5	Audience	7

2.6	Overview	8

2.7	Known Limitations	8

3	Business Processes	9

3.1	Overview	9

3.2	Common Topologies	9

3.2.1	TV and STB	9

3.2.2	TV, STB and IAP	10

3.3	Dealing with a Sound Bar or Surround Sound Receiver	11

3.3.1	Audio System Solution 1a	11

3.3.2	Audio System Solution 2a	12

3.4	Adding a Game Console	13

3.4.1	Game Console – 1	13

3.4.2	Game Console -2	14

3.5	Other Playback Devices	15

3.5.1	TV, STB, IAP and Playback Device	15

3.5.2	TV, STB and Playback Device	16

3.5.3	TV, Audio System, STB, IAP and Playback Device	17

3.5.4	Playback Device in Sound Bar Topology 2a	18

4	HDMI Consumer Electronic Control Functions	19

4.1	Overview	19

4.2	Roles	19

4.2.1	Television (TV)	19

4.2.2	Intelligent Auxiliary Panel (IAP)	19

4.2.3	Set-Top Box (STB) Master	19

4.2.4	Set-Top Box (STB) Slave	19

4.2.5	Playback Device (DVD, Blu-ray, Solid State)	20

4.2.6	Audio System	20

4.3	CEC Messages	20

4.4	HDMI Remote Control Pass-through Codes	25

5	Vendor Specific Consumer Electronic Control Functions	30

5.1	HTNG Vendor Specific Messages	30

5.2	HTNG Vendor Specific Messages – Mandatory and Optional Functions	44

5.3	HTNG Vendor Specific Remote Control Codes	48

6	Appendices	51

6.1	Glossary of Terms	51

6.2	Implementation Notes	52

6.2.1	Topologies Legend	52

6.3	Referenced Documents	53

# This Specification at a Glance

Typically, a separate remote control is required for each entertainment device in the guest room such as the TV, STB (Set Top Box), IAP (Intelligent Auxiliary Panel), DVD player and audio system. Also, this equipment normally does not function in a well integrated way making it difficult for a guest to use it when unfamiliar with how it should be operated.

This specification is intended for adoption in next generation equipment. It will allow CEC messages to be used to control all the entertainment equipment which will be connected together using only HDMI cables. Adoption of the specification by equipment manufacturers will create the next generation of ‘hospitality’ equipment models. This should be considered a migration from today’s smart port to CEC and hoteliers should be aware of this with regards to their purchasing decisions.

The Entertainment Device Control workgroup has defined a solution that has built upon existing HDMI standards to define an HTNG specification. This fully complies with the HDMI 1.3/1.4 specification and defines a hospitality profile that mandates a number of CEC messages to ensure interoperability of devices from multiple vendors in a hospitality environment. In addition HTNG has defined and owns a number of 'HDMI Vendor Specific' extensions which device vendors will be required to implement for compliance with the HTNG CEC specification.

Unfortunately, HTNG has as yet been unable to secure access to the CEC commands section of the HDMI 1.4 specification to allow that to be the basis of the HTNG specification. However, version 1.3 is substantially the same. The primary benefit of using version v1.4 would have been better CEC message descriptions. It is expected that manufacturers wishing to be certified to the HTNG CEC specification can design, manufacture and test based upon the v1.3 and v1.4 specifications. Those doing so will be HDMI members and will be able to access and benefit from the improved CEC descriptions in HDMI v1.4.

Key Design Decisions

- Consumer electronics standards were adopted in order to leverage current and future developments in consumer electronics while at the same time benefitting financially from the mass market economy of scale.

- HDMI CEC commands only - no proprietary, no legacy.

- HTNG has registered an OUI with IEEE in order to specify and own the hospitality vendor specific CEC commands for remote control functions.

- The design allows for systems with or without a Set Top Box (STB).

- Either the TV or STB can be the system master, as both are required to support conversion of all control commands to CEC messages which may be required by other devices in the room, even though they are not required by either the TV or STB themselves (these messages are passed through to the other equipment).

- A number of use cases have been defined.

- The technical specification has been tested, validated, and finalised by a number of manufacturers which participated in the workgroup:

  - TV: Philips, Samsung

  - Integrators: Swisscom, iBAHN, Deuromedia

  - Intelligent Auxiliary Panels: TeleAdapt, guestLINK

  - Playback Devices: Philips

  - Audio Systems: Philips

# Document Information

## Document History

| Version | Date | Author | Comments |
| --- | --- | --- | --- |
| 0.01 | 11 May 2011 | Jon Buenviaje | Inital draft of specification documentation |
| 0.02 | 12 May 2011 | Bryan Steele / Jon Buenviaje | Created content for Section 2 as well minor edits throughout the document |
| 0.03 | 26 May 2011 | Jon Buenviaje | Imported the “Vendor Specific CEC Commands and Remote Passthrough V0.20“ and “CEC Functions V0.96“ spreadsheets into Sections 4 and 5 as well as major formatting |
| 0.04 | 9 June 2011 | Team members | Imported section 1 – Spec at a glance |
| 0.05 | 5 July 2011 | Bryan Steele | Updated section 1 |
| 0.06 | 6 July 2011 | Team members | Updates in section 5.1 in response to Samsung’s feedback |
| 0.07 | 20 July 2011 | Team members | Additional updates in response to Samsung’s feedback |
| 0.08 | 20 July 2011 | Team members | Updated section 1 |
| 0.09 | 9 Aug 2011 | Team members | Sections 4.5 and 4.6 merged into sections 5.1 and 5.2. Updates to section introductions. |
| 0.10 | 17 Aug 2011 | Team members | Section 4.4 Updated (modified TV codes) |
| 0.11 | 31 Aug 2011 | Team members | Added Slave STB role in 4.3 and 4.4 |
| 0.12 | 7 Sept 2011 | Colin Corby / Jon Buenviaje | Master and Slave STB roles defined. 4.4 intro modified. 4.4 STB Slave role added. |
| 0.14 | 4 Oct 2011 | Jon Buenviaje | Incorporated Mark Laramie’s edits and partially reformatted section 5.1 |
| 0.15 | 4 Oct 2011 | Team members | Edited Section 5.1 and created section 5.3 |
| 0.16 | 25 Oct 2011 | Team members | Created placeholders for intro text and updated tables |
| 0.17 | 7 Nov 2011 | Team members | Reordered sections 5.2 and 5.3 and text updates from 1Nov 2011 teleconference |
| 0.18 | 8 Nov 2011 | Team members | Updated section 4.4 |
| 0.19 | 21 Nov 2011 | Jon Buenviaje | Imported tables of section 4.2, 4.3, 5.2, 5.3 and major formatting |
| 0.20 | 22 Nov 2011 | Team members | Updated tables sections 4 and 5 |
| 0.21 | 5 Dec 2011 | Team Members | Updated 4.3, 4.4, 5.1, 5.3 |
| 0.22 | 6 Dec 2011 | Team Members | Accepted Changes throughout document |
| 0.23 | 20 Dec 2011 | Team Members | Modified Overview section |
| 0.24 | 25 Jan 2012 | Team Members | Updated Document Purpose and Closed Captioning requirements |
| 1.00 | 23 Feb 2012 | Team Members | Ratified document |
| 1.10 | 2 March 2012 | Team members | Section 5.1: 0x00 and 0x81 updated notes; 0x10 and 0x65 updated Parameter descriptionx; |

If specification is modified by another workgroup please identify the workgroup under Author

## Document Purpose

The purpose of this document is to define a set of HDMI CEC commands to facilitate interoperability and consistent operation of entertainment devices in the guestroom.

This specification defines the “next generation” of hospitality entertainment devices. This would replace the current generation of equipment which utilizes proprietary serial interfaces to give a degree of interconnectivity but with interoperability challenges. HDMI CEC is an industry standard which is being adopted as the basis of this specification.

The intention will be to introduce certification based on this specification after a number of vendors have confirmed its suitability.

## Scope

Business Problem: Typically, a separate remote control is required for each device in the guest room such as the TV, Set Top Box, Intelligent Auxiliary Panel, DVD player and audio system. Also, this equipment normally does not function in a well-integrated way making it difficult for a guest to use it when unfamiliar with how it should be operated. Having multiple remote controls add to the clutter which can be found in a guest room. Manufacturers have not implemented HDMI CEC in a consistent way since little of the specification is mandated.

Solution: Entertainment equipment which can support the HTNG mandated message requirements within the HDMI CEC 1.3 specification. This would result in a single remote control which can be used to control multiple devices in the guest room which are connected together using HDMI.

## Relationship to Other Standards

HTNG was unable to utilize the HDMI 1.4 standard, which it wished to do, because of clearer description of CEC messages. Therefore, the HDMI 1.3 specification is referred to throughout this document. However, it is recognized that any company implementing HDMI CEC for their products will be members of HDMI and therefore have access to the HDMI 1.4 standard. HTNG strongly recommends that vendors refer to these more complete descriptions.

## Audience

This document is targeted at manufacturers of entertainment devices including:

- Television manufacturers

- Set-Top-Box manufacturers

- DVD / Blu-ray player manufacturers

- Audio system manufacturers

- Intelligent Auxiliary Panels manufacturers

- VOD system suppliers

In addition, the document is also targeted at hoteliers and consultants who specify and purchase entertainment equipment as well as system Integrators (design, specification, and build).

## Overview

Section 3 of this document describes the physical topologies considered.

Section 4 defines the roles of specific entertainment hardware and documents how the published HDMI standards should be applied for hospitality applications. More specifically any standard messages which are mandatory for compliance with this HTNG specification are identified.

Section 5 lists HTNG extensions to the published HDMI standards, these are in the form of vendor specific messages and vendor specific remote control codes.

The IEEE Organizationally Unique Identifier (OUI) for Hotel Technology Next Generation is 00-D3-8D (hex).

## Known Limitations

This document was originally based on the HDMI 1.3 specification. However it is recommended that implementers refer to the latest version of the HDMI specification. At the time of writing the latest version is 1.4.

HDMI specifications can be found here:  http://www.hdmi.org/manufacturer/specification.aspx

# Business Processes

## Overview

This section describes audio visual equipment connection guides to maximize the performance of HTNG compliant devices.

## Common Topologies

A use case is a more specific circumstance than what may be described in the Business Scenario. For example, there may be a Business Scenario for Check-In and a Use Case for Check-In via Kiosk.

### TV and STB

### TV, STB and IAP

## Dealing with a Sound Bar or Surround Sound Receiver

Configurations Expected in Premium Properties

### Audio System Solution 1a

### Audio System Solution 2a

## Adding a Game Console

Expected Connection Points For Game Consoles. This section assumes game consoles are provided by the hotel on a temporary basis upon guest request.

### Game Console – 1

### Game Console -2

## Other Playback Devices

Adding Playback Devices (e.g. DVD players, Blu-ray and other devices)

### TV, STB, IAP and Playback Device

### TV, STB and Playback Device

### TV, Audio System, STB, IAP and Playback Device

### Playback Device in Sound Bar Topology 2a

# HDMI Consumer Electronic Control Functions

## Overview

This specification assumed the use of HDMI 1.4

Logical Device Types

| Device | HDMI Logical Device Type | Notes |
| --- | --- | --- |
| TV | TV | When in lowest power mode, the TV must be able to route CEC messages to allow other devices (e.g. audio system) to function |
| IAP | Playback |  |
| STB | Tuner | Allocate logical address in reverse, 10 downwards |
| DVD | Playback |  |
| Audio | Audio |  |

## Roles

This specification defines the following roles:

### Television (TV)

A display device deployed in a guest room. This will comprise a plasma, LCD or LED panel with associated digital and or analog tuners. This specification assumes the use of a “hospitality” TV which implements specific features commonly required in hotels.

### Intelligent Auxiliary Panel (IAP)

A device which provides a range of additional guest inputs conveniently located in the guest room. It has a single HDMI output, non-digital inputs are converted to HDMI as required.

### Set-Top Box (STB) Master

A gateway device which provides additional services to the guest via the TV, for example movies on-demand, integration with other hotel systems like billing or room controls etc. An STB may be the master controller of the in-room system. This specification also recognizes that a remote control may be configured to send remote control codes directly to an STB. This would usually be an RF (non-directional) remote but in some situations could also be an IR device. In this configuration the STB would be expected to translate and forward received remote codes to the active source as HDMI CEC Remote Control Codes or HTNG Vendor Specific Remote Control Codes as required. It must also implement the HTNG mandated CEC messages and remote control codes for a master STB.

### Set-Top Box (STB) Slave

In standard HDMI CEC operation all remote control codes (either IR or RF) will be received by the TV and forwarded to the active source as an HDMI CEC Remote Control Code. A TV certified to this specification must also be able to forward HTNG Vendor Specific Remote Control Codes to the active source when requested. In this case, the STB has no role in remote control code translation and would be considered a slave. It must also implement the HTNG mandated CEC messages and remote control codes for a slave STB.

### Playback Device (DVD, Blu-ray, Solid State)

A device which allows a guest to playback pre-recorded content via the TV using HDMI.

### Audio System

A high quality audio system that is used to enhance the guest audio experience. TV audio output is redirected via the audio system. The audio system can be used independently of the TV. The audio system will connect via HDMI. Depending upon the HDMI implementation chosen the reverse audio channel may be implemented using with a discrete analog/digital link or using the reverse audio feature available in the HDMI 1.4 specification.

## CEC Messages

The messages listed below are described in detail in the High-Definition Multimedia Interface Specification Version 1.4. HDMI Adopters and Founders have full access to this document online at www.hdmi.org.

Only a few HDMI CEC messages are considered to be mandatory in the HDMI 1.4 specification, this can lead to interworking issues between devices. To reduce interworking issues and maximize supported functions devices conforming to the HTNG CEC requirements must implement the standard HDMI mandatory messages plus any standard messages shown as mandatory in the table below.

| KEY |  |
| --- | --- |
| S / I | Send / Initiate |
| R / F | Receive / Follow |
| X | Mandatory |
| D | Desirable |
| P | Prohibited |
| A | Feature Abort |
| - | Don't Care |

| Code | Feature / Message Description | Devices |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TV |  | IAP (Aux Panel) |  | STB Master |  | STB Slave |  | Playback Device |  | Audio (Digital Amp) |  |
|  |  | S/I | R/F | S/I | R/F | S/I | R/F | S/I | R/F | S/I | R/F | S/I | R/F |
| - | Polling Message | X (6) | X | X (6) | X | X (6) | X | X(6) | X | X (6) | X | X (6) | X |
| 0x00 | Feature Abort | X | X | X | X | X | X | X | X | X | X | X | X |
| 0x04 | Image View On | - | X | X (1) | - | X (1) | - | X(1) | - | X (1) | - | X (1) | - |
| 0x05 | Tuner Step Increment | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x06 | Tuner Step Decrement | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x07 | Tuner Device Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x08 | Give Tuner Device Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x09 | Record Off | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x0A | Record Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x0B | Record On | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x0D | Text View On | - | X | X (1) | - | X (1) | - | X(1) | - | X (1) | - | X (1) | - |
| 0x0F | Record TV Screen | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x1A | Give Deck Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x1B | Deck Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x32 | Set Menu Language | X | D | - | D | X | D | - | D | - | D | - | D |
| 0x33 | Clear Analogue Timer | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x34 | Set Analogue Timer | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x35 | Timer Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x36 | Standby | X (4) | X (5) | P | X (3) | X (4) | X (5) | P | X(3) | P | X (3) | P | X (3) |
| 0x41 | Play | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x42 | Deck Control | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x43 | Timer Cleared Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x44 | User Control Pressed | X(11) | - | - | X (11) | X(11) | - | - | X (11) | - | X (11) | - | X (11) |
| 0x45 | User Control Released | X(11) | - | - | X (11) | X(11) | - | - | X (11) | - | X (11) | - | X (11) |
| 0x46 | Give OSD Name | D | X | - | X | D | X | - | X | - | X |  | X |
| 0x47 | Set OSD Name | - | D | - | D | - | D | D | - | D | - | D | - |
| 0x64 | Set OSD String | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x67 | Set Timer Program Title | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x70 | System Audio Mode Request | D | - | - | - | D | - | - | - | - | - | - | X |
| 0x71 | Give Audio Status | - | - | - | - | - | - | - | - | - | - | - | X |
| 0x72 | Set System Audio Mode | - | X | - | - | - | X | - | - | - | - | - | X |
| 0x7A | Report Audio Status | - | - | - | - | - | - | - | - | - | - | X | - |
| 0x7D | Give System Audio Mode Status | - | - | - | - | - | - | - | - | - | - | - | X |
| 0x7E | System Audio Mode Status | - | - | - | - | - | - | - | - | - | - | X | - |
| 0x80 | Routing Change | X | X | X (2) | X(2) | X (2) | X(2) | X (2) | X(2) | X (2) | X(2) | X (2) | X(2) |
| 0x81 | Routing Information | - | - | X (2) | X(2) | X (2) | X(2) | X(2) | X(2) | X (2) | X(2) | X (2) | X(2) |
| 0x82 | Active Source | X | X | X | X | X | X | X | X | X | X | X | X |
| 0x83 | Give Physical Address | X | X | - | X | X | X | - | X | - | X | - | X |
| 0x84 | Report Physical Address | X | X | X | - | X | X | X | - | X | - | X | - |
| 0x85 | Request Active Source | X | X | - | X | X | X | - | X | - | X | - | X |
| 0x86 | Set Stream Path | X | - | - | X (2) | X | X (2) | - | X(2) | - | X (2) | - | X (2) |
| 0x87 | Device Vendor ID | - | X | - | X | - | X | - | X | - | X | - | X |
| 0x89 | Vendor Command | P | P | P | P | P | P | P | P | P | P | P | P |
| 0x8A | Vendor Remote Button Down | X(7) | - | - | X(7) | X(7) | - | - | X(7) | - | X(7) | - | X(7) |
| 0x8B | Vendor Remote Button Up | X(7) | - | - | X(7) | X(7) | - | - | X(7) | - | X(7) | - | X(7) |
| 0x8C | Give Device Vendor ID | X | X | X | X | X | X | X | X | X | X | X | X |
| 0x8D | Menu Request | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x8E | Menu Status | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x8F | Give Device Power Status | - | X | - | X | - | X | - | X | - | X | - | X |
| 0x90 | Report Power Status | X | - | X | - | X |  | X | - | X | - | X | - |
| 0x91 | Get Menu Language | - | X | D | - | - | X | D | - | D | - | D | - |
| 0x92 | Select Analogue Service | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x93 | Select Digital Service | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x97 | Set Digital Timer | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x99 | Clear Digital Timer | - | - | - | - | - | - | - | - | - | - | - | - |
| 0x9A | Set Audio Rate | - | - | - | D | - | D | - | D | - | D | - | D |
| 0x9D | Inactive Source |  | X(10) | X |  | X |  | X | - | X | - | X | - |
| 0x9E | CEC Version | X | X | X |  | X | X | X | - | X | - | X | - |
| 0x9F | Get CEC Version | X | X | - | X | X | X | - | X | - | X | - | X |
| 0xA0 | Vendor Command With ID | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) | X(7) |
| 0xA1 | Clear External Timer | - | - | - | - | - | - | - | - | - | - | - | - |
| 0xA2 | Set External Timer | - | - | - | - | - | - | - | - | - | - | - | - |
| 0xA3 | Report Short Audio Descriptor | - | - | - | - | - | - | - | - | - | - | X | - |
| 0xA4 | Request Short Audio Descriptor | - | - | - | - | - | - | - | - | - | - | - | X |
| 0xC0 | Initiate ARC | - | X (8) | - | - | - | - | - | - | - | - | - | - |
| 0xC1 | Report ARC Initiated | - | X (8) | - | - | - | - | - | - | - | - | - | - |
| 0xC2 | Report ARC Terminated | - | X (8) | - | - | - | - | - | - | - | - | - | - |
| 0xC3 | Request ARC Initiation | D (8) | - | - | X (8) | - | X (8) | - | X (8) | - | X (8) | - | X (8) |
| 0xC4 | Request ARC Termination | D (8) | - | - | X (8) | - | X (8) | - | X (8) | - | X (8) | - | X (8) |
| 0xC5 | Terminate ARC | - | X (8) | D (8) | - | D (8) | - | D (8) | - | D (8) | - | D (8) | - |
| 0xF8 | CDC Message | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) | X (9) |

1                As a minimum these devices should implement one of Image View On or Text View On.

2                Routing Change and Set Stream Path must be implemented if the device can act as an HDMI switch (has >1 HDMI input).

3                If the device has a standby mode (this may be mandated in some regions) then this message must be implemented.

4                The TV or STB may transmit either directly addressed or broadcast Standby messages as they choose.

5                The action performed by an STB that receives a Standby message will be implementation dependent.

6                A device may poll logical addresses no more than once every 30 seconds.

7                An HTNG OUI will be assigned to allow HTNG specific RC codes to be sent to the STB. See worksheet 4.6 HTNG Remote Control Extension Codes

8                Only applies if a device supports HDMI 1.4 and ARC.

9                Applies if a device supports HDMI 1.4.

10              If the TV receives an inactive source from any device other than the STB and an STB is present then the TV should switch to the previous source.

11              See 4.4 HDMI Remote Control Pass-through Codes, for definitions of which devices implement which RC codes.

## HDMI Remote Control Pass-through Codes

This section identifies which of the standard HDMI CEC remote control must be implemented for compliance with this HTNG specification.

In the HDMI-CEC specification it is the function of the TV to physically receive the signal from the remote control and to translate this signal into an HDMI-CEC message, the content of which describes the button pressed or released by the user, and then forward the message, if needed, to the active device.

This specification recognizes that RF based remote controls may be used and linked to the STB instead of the TV. In this case the STB would need to perform the translation normally handled by the TV.

The remote control codes listed below are the standard HDMI-CEC remote control codes.

Devices that receive the raw remote control codes (TV or STB) must be able to translate the mandated buttons to the corresponding HDMI-CEC remote control code and forward as necessary.

Devices that receive the HDMI-CEC remote control codes from the translator must provide functionality to implement the mandatory codes and should consider the benefit or otherwise of implementing optional codes.

HTNG favors the use of deterministic codes where they are available (e.g. 0x60 – 0x6D) and specifically excludes the use of equivalent non-deterministic codes. Where a non-deterministic code has no deterministic equivalent, the non-deterministic code is use instead.

| Legend |  |
| --- | --- |
| Mandatory RC Key Code | X |
| Optional RC Key Code | O |
| Prohibited | P |
| Don’t Care | - |

| Remote Control Pass-through Code | Remote Control Pass-through Code Description | Remote Control Translator |  | Remote Control CEC Message Receiver |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TV | STB Master | TV | STB Master | STB Slave | IAP | Optical Only Playback Device | Multi-media Cloud Device * | Audio ** |
| 0x00 | Select | X | X | X | X | X | O | X | X | X |
| 0x01 | Up | X | X | X | X | X | O | X | X | X |
| 0x02 | Down | X | X | X | X | X | O | X | X | X |
| 0x03 | Left | X | X | X | X | X | O | X | X | X |
| 0x04 | Right | X | X | X | X | X | O | X | X | X |
| 0x05 | Right-Up | O | O | O | O | O | O | O | O | O |
| 0x06 | Right-Down | O | O | O | O | O | O | O | O | O |
| 0x07 | Left-Up | O | O | O | O | O | O | O | O | O |
| 0x08 | Left-Down | O | O | O | O | O | O | O | O | O |
| 0x09 | Root Menu | X | X | O | O | X | O | X | X | O |
| 0x0A | Setup Menu | X | X | O | O | O | O | O | O | O |
| 0x0B | Contents Menu | O | O | O | O | O | O | O | O | O |
| 0x0C | Favorite Menu | O | O | O | O | O | O | O | O | O |
| 0x0D | Exit | X | X | O | O | O | O | O | O | O |
| 0x0E - 0x1F | Reserved | P | P | P | P | P | P | P | P | P |
| 0x20 - 0x29 | Numbers 0-9 | X | X | X | X | X | O | O | O | O |
| 0x2A | Dot | X | X | X | O | X | - | - | O | O |
| 0x2B | Enter† | X | X | O | O | O | O | O | O | O |
| 0x2C | Clear | O | O | O | O | O | O | O | O | O |
| 0x2D - 0x2E | Reserved | P | P | P | P | P | P | P | P | P |
| 0x2F | Next Favorite | O | O | O | O | O | O | O | O | O |
| 0x30 | Channel Up | X | X | X | X | - | - | - | O | O |
| 0x31 | Channel Down | X | X | X | X | X | - | - | O | O |
| 0x32 | Previous Channel | X | X | X | X | X | - | - | O | O |
| 0x33 | Sound Select | O | O | O | O | - | - | - | - | O |
| 0x34 | Input Select | - | - | - | - | - | - | - | - | - |
| 0x35 | Display Information | X | X | O | O | O | O | O | O | O |
| 0x36 | Help | O | O | O | O | O | O | O | O | O |
| 0x37 | Page Up | O | O | O | O | O | O | O | O | O |
| 0x38 | Page Down | O | O | O | O | O | O | O | O | O |
| 0x39-0x3F | Reserved | P | P | P | P | P | P | P | P | P |
| 0x40 | Power | - | - | - | - | - | - | - | - | - |
| 0x41 | Volume Up | X | X | X | X | O | - | - | - | X |
| 0x42 | Volume Down | X | X | X | X | O | - | - | - | X |
| 0x43 | Mute | - | - | - | - | - | - | - | - | - |
| 0x44 | Play | - | - | - | - | - | - | - | - | - |
| 0x45 | Stop | - | - | - | - | - | - | - | - | - |
| 0x46 | Pause | - | - | - | - | - | - | - | - | - |
| 0x47 | Record | - | - | - | - | - | - | - | - | - |
| 0x48 | Rewind | X | X | O | O | X | O | X | X | O |
| 0x49 | Fast Forward | X | X | O | O | X | O | X | X | O |
| 0x4A | Eject | X | X | O | O | O | O | O | O | O |
| 0x4B | Forward | X | X | O | O | O | O | O | O | - |
| 0x4C | Backward | X | X | O | O | O | O | O | O | - |
| 0x4D | Stop-Record | - | - | - | - | - | - | - | - | - |
| 0x4E | Pause-Record | - | - | - | - | - | - | - | - | - |
| 0x4F | Reserved | P | P | P | P | P | P | P | P | P |
| 0x50 | Angle | O | O | O | O | O | - | O | O | - |
| 0x51 | Sub picture | O | O | O | O | - | - | - | O | - |
| 0x52 | Video on Demand | O | O | O | O | - | - | - | O | - |
| 0x53 | Electronic Program Guide | X | X | O | O | - | - | - | - | - |
| 0x54 | Timer Programming | X | X | O | O | - | - | - | - | - |
| 0x55 | Initial Configuration | - | - | - | - | - | - | - | - | - |
| 0x56 - 0x5F | Reserved | P | P | P | P | P | P | P | P | P |
| 0x60 | Play Function | X | O | O | O | X | O | X | X | O |
| 0x61 | Pause-Play Function‡ | X | O | O | O | X | O | X | X | O |
| 0x62 | Record Function | X | O | O | O | O | - | - | O | O |
| 0x63 | Pause-Record Function | - | - | - | - | - | - | - | - | - |
| 0x64 | Stop Function | X | X | O | O | X | O | X | X | O |
| 0x65 | Mute Function | X | X | X | X | O | - | - | - | X |
| 0x66 | Restore Volume Function | O | O | O | O | O | - | - | - | O |
| 0x67 | Tune Function | - | - | - | - | - | - | - | - | - |
| 0x68 | Select Media Function | X | X | O |  | - | - | - | O | - |
| 0x69 | Select A/V Input Function | X | X | X | X | - | O | - | - | O |
| 0x6A | Select Audio Input Function | O | O | O | O | - | O | - | - | O |
| 0x6B | Power Toggle Function | X | X | X | X | O | O | O | O | O |
| 0x6C | Power Off Function | X | X | X | X | O | O | O | O | O |
| 0x6D | Power On Function | X | X | X | X | O | O | O | O | O |
| 0x6E - 0x70 | Reserved | P | P | P | P | P | P | P | P | P |
| 0x71 | F1 (Blue) | X | X | O | O | O | O | O | O | O |
| 0x72 | F2 (Red) | X | X | O | O | O | O | O | O | O |
| 0x73 | F3 (Green) | X | X | O | O | O | O | O | O | O |
| 0x74 | F4 (Yellow) | X | X | O | O | O | O | O | O | O |
| 0x75 | F5 | O | O | O | O | O | O | O | O | O |
| 0x76 | Data (Teletext for Japan) | O | O | O | O | - | - | - | O | - |
| 0x77 - 0xFF | Reserved | P | P | P | P | P | P | P | P | P |

- * 	Blu-ray, GoogleTV, etc.

**	If Audio system includes any Optical Playback, it must also obey the Optical Only Playback Device Column, with conflicts becoming optional

†	“Enter” and “Select” have the same function so the “Select” button was chosen to be mandatory and “Enter” was therefore deemed optional.

‡	Some systems have separate “Play” and “Pause” codes. Some use the same code as a “Play/Pause” toggle

# Vendor Specific Consumer Electronic Control Functions

## HTNG Vendor Specific Messages

CEC OpCode - 0xA0 (Vendor Command with ID) Vendor ID must be HTNG OUI (00-D3-8D).

If the function is not supported on the TV or STB then the TV or STB should return Feature Abort. The Feature Abort command should contain the unsupported message. If the function is supported then the messaging is mandatory

| HTNG OpCode | 0x00 |
| --- | --- |
| Description | Tune to 1-Part channel number |
| Parameter Length | 3 bytes |
| Parameter Description | First Byte – 00 Air, 01 Cable, 10 Not Used, 11 Source Not Specified. Second and Third Bytes Channel number 0x0000-0xFFFF |
| Action on Receipt | Select the defined channel on TV or STB |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | No dot or dash in the channel number. Feature abort message can be returned for 11 Source Not Specified |

| HTNG OpCode | 0x01 |
| --- | --- |
| Description | Tune to 2-Part channel number |
| Parameter Length | 4 bytes |
| Parameter Description | First Byte – 00 Air, 01 Cable, 10 Satellite, 11 Source Not Specified. Second byte – RF or Major Channel number 0x00-0xFF Third and Fourth Bytes Minor or MPEG Channel number 0x0000-0xFFFF |
| Action on Receipt | Select the defined channel on TV or STB |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Dot or Dash included in the channel number |

| HTNG OpCode | 0x02 |
| --- | --- |
| Description | Input select AV (Composite) |
| Parameter Length | 2 bytes |
| Parameter Description | AV input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined composite AV input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x03 |
| --- | --- |
| Description | Input select PC |
| Parameter Length | 2 bytes |
| Parameter Description | PC input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined PC input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x04 |
| --- | --- |
| Description | Input select HDMI |
| Parameter Length | 2 bytes |
| Parameter Description | HDMI input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined HDMI input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x05 |
| --- | --- |
| Description | Input select Component |
| Parameter Length | 2 bytes |
| Parameter Description | Component input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined component AV input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x06 |
| --- | --- |
| Description | Input select DVI |
| Parameter Length | 2 bytes |
| Parameter Description | DVI input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined DVI input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x07 |
| --- | --- |
| Description | Input select DisplayPort |
| Parameter Length | 2 bytes |
| Parameter Description | DP input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined DP input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x08 |
| --- | --- |
| Description | Input select USB |
| Parameter Length | 2 bytes |
| Parameter Description | USB input no 0x0000 - 0x000F |
| Action on Receipt | Select the defined USB input |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Mandatory only if input available on device. |

| HTNG OpCode | 0x09 - 0x0F |
| --- | --- |
| Description | Input select Reserved |
| Parameter Length | - |
| Parameter Description | - |
| Action on Receipt | - |
| Directly Addressed or Broadcast | - |
| Notes | - |

| HTNG OpCode | 0x10 |
| --- | --- |
| Description | Set Default Power On Input Source |
| Parameter Length | 5 Bytes |
| Parameter Description | First Byte - 1-Part TV channel no. (0x00), 2-Part TV channel no. (0x01), Composite AV (0x02), PC (0x03), HDMI (0x04), Component (0x05), DVI (0x06), DP (0x07), USB (0x08). Second Byte defines channel type for 1 and 2-Part channel nos.– 00 Air, 01 Cable, 10 Satellite, 11 Source Not Specified. For all except 2-Part channel nos.: Third and Fourth bytes define channel number for 1-Part channel no. (0x0000-0xFFFF), or input number (0x0000-0x000F) for external inputs and Fifth Byte is don’t care For 2-Part channel nos.: Third byte RF or Major Channel number 0x00-0xFF, Fourth and Fifth byte for Minor Channel number |
| Action on Receipt | Select the default input source |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x11 - 0x1F |
| --- | --- |
| Description | Reserved |
| Parameter Length | - |
| Parameter Description | - |
| Action on Receipt | - |
| Directly Addressed or Broadcast | - |
| Notes | - |

| HTNG OpCode | 0x20 |
| --- | --- |
| Description | Set TV Speakers |
| Parameter Length | 1 byte |
| Parameter Description | TV speaker state, Off (0x00), On (0x01) |
| Action on Receipt | Turn TV Speakers On/Off |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Changed from a toggle function to parameter driven off/on. |

| HTNG OpCode | 0x21 |
| --- | --- |
| Description | Set Digital Audio |
| Parameter Length | 1 byte |
| Parameter Description | Digital audio state, Off (0x00), On (0x01) |
| Action on Receipt | Turn Digital Audio On/Off |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Changed from a toggle function to parameter driven off/on. |

| HTNG OpCode | 0x22 |
| --- | --- |
| Description | Set Analog Audio |
| Parameter Length | 1 byte |
| Parameter Description | Analogue audio state, Off (0x00), On (0x01) |
| Action on Receipt | Turn Analog Audio On/Off |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Changed from a toggle function to parameter driven off/on. |

| HTNG OpCode | 0x23 |
| --- | --- |
| Description | Set Default Power On Volume |
| Parameter Length | 1 byte |
| Parameter Description | Default Power On volume setting, 0x00-0xFF |
| Action on Receipt | Set Default Power On Volume |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x24 |
| --- | --- |
| Description | Set Maximum Volume |
| Parameter Length | 1 byte |
| Parameter Description | Maximum volume setting, 0x00-0xFF |
| Action on Receipt | Set Maximum Volume |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x25 |
| --- | --- |
| Description | Set Minimum Volume |
| Parameter Length | 1 byte |
| Parameter Description | Minimum volume setting, 0x00-0xFF |
| Action on Receipt | Set Minimum Volume |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x26 – 0x3F |
| --- | --- |
| Description | Reserved for Audio |
| Parameter Length | - |
| Parameter Description | - |
| Action on Receipt | - |
| Directly Addressed or Broadcast | - |
| Notes | - |

| HTNG OpCode | 0x40 |
| --- | --- |
| Description | Set Blue Screen |
| Parameter Length | 1 byte |
| Parameter Description | TV screen state, Black (0x00), Blue (0x01) |
| Action on Receipt | Set screen color when no video signal. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x41 |
| --- | --- |
| Description | Set Brightness |
| Parameter Length | 1 byte |
| Parameter Description | TV display brightness setting, 0x00-0xFF |
| Action on Receipt | Set Brightness |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x42 |
| --- | --- |
| Description | Set Color |
| Parameter Length | 1 byte |
| Parameter Description | TV display color setting, 0x00-0xFF |
| Action on Receipt | Set Color |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x43 |
| --- | --- |
| Description | Set Contrast |
| Parameter Length | 1 byte |
| Parameter Description | TV display contrast setting, 0x00-0xFF |
| Action on Receipt | Set Contrast |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x44 |
| --- | --- |
| Description | Set Sharpness |
| Parameter Length | 1 byte |
| Parameter Description | TV display sharpness setting, 0x00-0xFF |
| Action on Receipt | Set Sharpness |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x45 |
| --- | --- |
| Description | Set Hue |
| Parameter Length | 1 byte |
| Parameter Description | TV display hue setting, 0x00-0xFF |
| Action on Receipt | Set Hue |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x46 |
| --- | --- |
| Description | Set LED Backlight |
| Parameter Length | 1 byte |
| Parameter Description | TV display LED backlight setting, 0x00-0xFF |
| Action on Receipt | Set LED Backlight |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x47 |
| --- | --- |
| Description | TV OSD Control |
| Parameter Length | 1 byte |
| Parameter Description | TV OSD setting, Off (0x00), On (0x01) |
| Action on Receipt | Set OSD Status |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes |  |

| HTNG OpCode | 0x48 |
| --- | --- |
| Description | Set Audio Only Display |
| Parameter Length |  |
| Parameter Description | Audio Only Display, Off (0x00), On (0x01) |
| Action on Receipt | If On and no video signal (audio only) then turn display off. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes |  |

| HTNG OpCode | 0x49 – 0x5F |
| --- | --- |
| Description | Reserved for Display |
| Parameter Length | - |
| Parameter Description | - |
| Action on Receipt | - |
| Directly Addressed or Broadcast | - |
| Notes | - |

| HTNG OpCode | 0x60 |
| --- | --- |
| Description | Set Date |
| Parameter Length | 8 Bytes |
| Parameter Description | ASCII in format DDMMYYYY. |
| Action on Receipt | Set the system date. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x61 |
| --- | --- |
| Description | Set Date Display Format |
| Parameter Length | 1 Byte |
| Parameter Description | MM-DD-YYYY (0x00), DD-MM-YYYY (0x01) |
| Action on Receipt | Set the date display format |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x62 |
| --- | --- |
| Description | Set Time |
| Parameter Length | 6 Bytes |
| Parameter Description | 6 ASCII bytes, always in format HHMMSS |
| Action on Receipt | Set the system time |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x63 |
| --- | --- |
| Description | Set Clock Brightness - Standby |
| Parameter Length | 1 Byte |
| Parameter Description | Brightness of the clock when in standby, 0x00-0xFF |
| Action on Receipt | Set the brightness of the clock when in standby |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x64 |
| --- | --- |
| Description | Set Clock Brightness - On |
| Parameter Length | 1 Byte |
| Parameter Description | Brightness of the clock when system on, 0x00-0xFF |
| Action on Receipt | Set the brightness of the clock when system on |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x65 |
| --- | --- |
| Description | LED Control |
| Parameter Length | - 1 Byte |
| Parameter Description | 0x00 is default; 0x01 is on; 0x02 is off |
| Action on Receipt | LED blink messages to indicate system status |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Default action is return LED control to device with LED |

| HTNG OpCode | 0x66 |
| --- | --- |
| Description | Lock TV Power Button |
| Parameter Length | 1 Byte |
| Parameter Description | TV power button lock status, Off (0x00), On (0x01) |
| Action on Receipt | If on then lock (disable) TV Power button |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x67 |
| --- | --- |
| Description | Lock TV Volume Buttons |
| Parameter Length | 1 Byte |
| Parameter Description | TV volume button lock status |
| Action on Receipt | If on then lock (disable) TV Volume button |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x68 |
| --- | --- |
| Description | Lock TV Channel Buttons |
| Parameter Length | 1 Byte |
| Parameter Description | TV channel buttons lock status, Off (0x00), On (0x01) |
| Action on Receipt | If on then lock (disable) TV Channel buttons |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x69 |
| --- | --- |
| Description | Lock TV Input Buttons |
| Parameter Length | 1 Byte |
| Parameter Description | TV input buttons loc status, Off (0x00), On (0x01) |
| Action on Receipt | If on then lock (disable) TV input buttons |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x6A |
| --- | --- |
| Description | Lock All Other TV Buttons |
| Parameter Length | 1 Byte |
| Parameter Description | Lock status of other TV buttons, Off (0x00), On (0x01) |
| Action on Receipt | If on then lock (disable) other (all except Power, Volume, Channel and Input) TV buttons |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x6B |
| --- | --- |
| Description | Everything Including Power Locked |
| Parameter Length | 1 Byte |
| Parameter Description | Lock status of all TV buttons, Off (0x00), On (0x01) |
| Action on Receipt | If on then lock (disable) all TV buttons |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x6C |
| --- | --- |
| Description | Everything Except Power Locked |
| Parameter Length | 1 Byte |
| Parameter Description | Lock status of all TV buttons except the power, Off (0x00), On (0x01) |
| Action on Receipt | If on then lock (disable) all TV buttons except Power. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x6D |
| --- | --- |
| Description | Hotel Mode |
| Parameter Length | 2 Bytes |
| Parameter Description | Byte 1, Hotel mode status, Off (0x00), On (0x01), byte 2 as defined by individual manufacturers |
| Action on Receipt | Turn hotel mode on or off and allow manufacturer specific options. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x6E - 0x7F |
| --- | --- |
| Description | Reserved for commercial use |
| Parameter Length | - |
| Parameter Description | - |
| Action on Receipt | - |
| Directly Addressed or Broadcast | - |
| Notes | - |

| HTNG OpCode | 0x80 |
| --- | --- |
| Description | Set Power Saving Profile |
| Parameter Length | 2 Bytes |
| Parameter Description | Byte 1, Power Saving Profile, Off (0x00), On (0x01), byte 2 as defined by individual manufacturers |
| Action on Receipt | Turn power saving profile on or off and allow manufacturer specific options. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x81 |
| --- | --- |
| Description | Set Sleep Timer |
| Parameter Length | 1 byte |
| Parameter Description | 0x00 - 0xFF (minutes) |
| Action on Receipt | Set number of minutes before system goes to sleep |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | 00 means sleep timer is inactive; 01-FF means that the action will take place after the set time. |

| HTNG OpCode | 0x82 |
| --- | --- |
| Description | Set Wakeup Time |
| Parameter Length | 4 bytes |
| Parameter Description | 4 ASCII bytes, always in format HHMM |
| Action on Receipt | Set time when system is to "wake up" |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x83 |
| --- | --- |
| Description | Set Auto-off Time |
| Parameter Length | 4 bytes |
| Parameter Description | 4 ASCII bytes, always in format HHMM |
| Action on Receipt | Set Time when system is to automatically switch off |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x84 |
| --- | --- |
| Description | Set Wakeup Source |
| Parameter Length | 4 bytes |
| Parameter Description | First Byte - 1-Part TV channel no. (0x00), 2-Part TV channel no. (0x01), Composite AV (0x02), PC (0x03), HDMI (0x04), Component (0x05), DVI (0x06), DP (0x07), USB (0x08). Second Byte defines channel type for 1 and 2-Part channel nos.– 00 Air, 01 Cable, 10 Satellite, 11 Source Not Specified. For all except 2-Part channel nos.: Third and Fourth bytes define channel number for 1-Part channel no. (0x0000-0xFFFF), or input number (0x0000-0x000F) for external inputs and Fifth Byte is don’t care For 2-Part channel nos.: Third byte RF or Major Channel number 0x00-0xFF Fourth and Fifth Bytes Minor or MPEG Channel number 0x0000-0xFFFF |
| Action on Receipt | Set Input source which is to be selected for "wake up" |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x85 |
| --- | --- |
| Description | Set initial wakeup volume |
| Parameter Length | 2 bytes |
| Parameter Description | Byte 1 volume, 0x00 - 0xFF byte 2 ramp-up time in minutes 0x00-0xFF |
| Action on Receipt | Set volume of system when it starts at "wake up" and ramp up time. |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | Max value 255, scale as required |

| HTNG OpCode | 0x86 |
| --- | --- |
| Description | Clear All Sleep/Wake Settings |
| Parameter Length | - |
| Parameter Description | - |
| Action on Receipt | All sleep/wake settings are cleared on the system |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x87 |
| --- | --- |
| Description | Keyboard Input |
| Parameter Length | 2 bytes |
| Parameter Description | - TBD |
| Action on Receipt |  |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | - |

| HTNG OpCode | 0x88 |
| --- | --- |
| Description | Global Direct (Frequency) Tune Command |
| Parameter Length | 8 bytes |
| Parameter Descriptions Parameter 1: Signal Type Parameter 2: Frequency (in steps of 62.5 kHz) Parameter 3: Service ID (Valid only for Digital Channels) Parameter 4: Modulation Type / Optional Parameters (Valid only for DVB, ISDB, DTMB) Parameter 5: Symbol rate (Valid only for DVB) (where 0 – 50000 represents 2.0000 – 7.0000) | 1st Byte Lower Nybble – Channel Type 0 – Auto 1 – Analog Antenna 2 – Analog Cable 3 – Digital Antenna 4 – Digital Cable 5 – Reserved 6 – Reserved Bit 4 0 = A/V program 1 = Audio only (radio) program Upper Nybble – System 0 – PAL B/G 1 – PAL I 2 – PAL D/K 3 – PAL M 4 – PAL N 5 – SECAM B/G 6 – SECAM D/K 7 – SECAM L 8 – NTSC M (9 – F) Reserved 2nd byte – upper 8 bits of frequency 3rd byte – lower 8 bits of frequency 4th byte – upper 8 bits of Service ID 5th byte – lower 8 bits of Service ID 6th byte Lower Nybble – Modulation Type 0 – Auto 1 – QPSK 2 – QAM16 3 – QAM32 4 – QAM64 5 – QAM128 6 – QAM256 7 – DQPSK 8 – F Reserved Upper Nybble – Other optional parameters Bit 3 0 = Symbol Rate is set to AUTO, ignore Parameter 5 1 = Symbol Rate follows Parameter 5 (other bits in Upper Nybble ignored for OpCode 88) 7th byte – High byte of Symbol Rate 8th byte – Low byte of Symbol Rate (example: 100 represents 2.0100) |
| Action on Receipt | Select the defined channel on TV or STB |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | TV shall use the parameters as configured in the set command. No fall-back mechanism shall be used even though the parameters are not correct. For example, if the channel is broadcasted using SECAM L set command direct tunes a channel using PAL B, no channel shall be tuned and a blank screen is expected in this scenario. TV shall honor this command even if the currently tuned extension is non-tuner. TV shall then switch to tuner and perform the direct tuning as per descripted. |

| HTNG OpCode | 0x89 |
| --- | --- |
| Description | Global Direct (Tuner Channel) Tune Command |
| Parameter Length | 3 bytes |
| Parameter Descriptions Parameter 1: Signal Type Parameter 2: Channel Number (converted to Hexadecimal) | 1st Byte Lower Nybble – Channel Type Bits (0-2) 0 – Auto 1 – Analog Antenna 2 – Analog Cable 3 – Digital Antenna 4 – Digital Cable 5 – Reserved 6 – Satellite 7 – Reserved Bit 3 0 = A/V program 1 = Audio only (radio) program Upper Nybble Reserved 2nd byte – Most significant byte of channel number 3rd byte – Least significant byte of channel number (i.e. decimal channel no. 427 = 01 AB) |
| Action on Receipt | Select the defined channel on TV or STB |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes |  |

| HTNG OpCode | 0x8A |
| --- | --- |
| Description | Extended Global Direct (Frequency) Tune Command |
| Parameter Length | 13 bytes |
| Parameter Descriptions Parameter 1: Signal Type Parameter 2: Frequency (in steps of 62.5 kHz) Parameter 3: Service ID (Valid only for DVB, ISDB, DTMB) Parameter 4: Modulation Type / Optional Parameters (Valid only for DVB, ISDB, DTMB) Bit 2 is for TSID (non-DVB-T2) or PLP (DVB-T2) Parameter 5: ONID (Valid only for DVB) Parameter 6: NID (Valid only for DVB) Parameter 7: TSID or PLP (Valid only for DVB) (TSID if non DVB-T2; PLP if DVB-T2) Parameter 8: Symbol rate (Valid only for DVB) (where 0 – 50000 represents 2.0000 – 7.0000) | 1st Byte Lower Nybble – Channel Type 0 – Auto 1 – Analog Antenna 2 – Analog Cable 3 – DVB-T, ISDB-T, DTMB 4 – DVB-C 5 – DVB-T2 6 – Reserved 7 – Reserved Bit 4 0 = A/V program 1 = Audio only (radio) program Upper Nybble – System 0 – PAL B/G 1 – PAL I 2 – PAL D/K 3 – PAL M 4 – PAL N 5 – SECAM B/G 6 – SECAM D/K 7 – SECAM L 8 – NTSC M (9 – F) Reserved 2nd byte – upper 8 bits of frequency 3rd byte – lower 8 bits of frequency 4th byte – upper 8 bits of Service ID 5th byte – lower 8 bits of Service ID 6th byte Lower Nybble – Modulation Type 0 – Auto 1 – QPSK 2 – QAM16 3 – QAM32 4 – QAM64 5 – QAM128 6 – QAM256 7 – DQPSK 8 – F Reserved Upper Nybble – Other optional parameters Bit 0 0 = ONID is set to AUTO, ignore Parameter 5 1 = ONID follows Parameter 5 Bit 1 0 = NID is set to AUTO, ignore Parameter 6 1 = NID follows Parameter 6 Bit 2 0 = TSID/PLP is set to AUTO, ignore Parameter 7 1 = TSID/PLP follows Parameter 7 Bit 3 0 = Symbol rate is set to AUTO, ignore Parameter 8 1 = Symbol rate follows Parameter 8 Bits 4 – 7 Reserved 7th byte – High byte of ONID 8th byte – Low byte of ONID 9th byte – High byte of NID 8th byte – Low byte of NID 10th byte – High byte of TSID/PLP 11th byte – Low byte of TSID/PLP 12th byte – High byte of Symbol Rate 13th byte – Low byte of Symbol Rate (example: 100 represents 2.0100) |
| Action on Receipt | Select the defined channel on TV or STB |
| Directly Addressed or Broadcast | Directly Addressed |
| Notes | TV shall use the parameters as configured in the set command. No fall-back mechanism shall be used even though the parameters are not correct. For example, if the channel is broadcasted using 64-QAM and the set command direct tunes a channel using 32-QAM, no channel shall be tuned and a blank screen is expected in this scenario. TV shall honor this command even if the currently tuned extension is non-tuner. TV shall then switch to tuner and perform the direct tuning as per descripted. |

## HTNG Vendor Specific Messages – Mandatory and Optional Functions

In order to ensure the necessary guest functionalities are both available and interoperable, the HTNG has designated certain of the HTNG Vendor Specific messages as mandatory.  Within these mandatory messages, there are two subcategories:

X    denotes those HTNG Vendor Specific CEC messages that must be supported in any condition.  The designated device must always support this function and must always also support the HTNG Vendor Specific messages associated with this function.  For example, a TV must always support channel tuning and must always also support the HTNG vendor specific message to tune a channel.

X(1) denotes those HTNG Vendor Specific CEC messages that must be supported only if the device supports that function.  The designated device is not required to support the particular function, but if it does support the function then it must also support the HTNG Vendor Specific message associated with the function. For example, a TV is not required to have a PC input, but if it does have a PC input then it must support the HTNG Vendor Specific message to select the PC input.

Any device receiving an HTNG vendor Specific message that it does not support should behave the same as prescribed for any unsupported HDMI CEC message:  that is return a Feature Abort message containing the relevant reason.

| Legend |  |
| --- | --- |
| X | Mandatory support of both Function and its associated CEC Messaging |
| X(1) | Support of the Function is Optional; however IF the Function is supported, then support for its associated CEC Messaging is Mandatory. |
| O | Optional |
| P | Prohibited Functions |
| - | Not Applicable |

| Code | Feature / Message Description | Devices |  |  |  |  |  |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TV |  | IAP (Aux Panel) |  | STB Master |  | STB Slave |  | Playback Device |  | Audio (Digital Amp) |  |  |
|  |  | S/I | R/F | S/I | R/F | S/I | R/F | S/I | R/F | S/I | R/F | S/I | R/F |  |
|  | Channel Tuning |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x00 | Tune 1-Part Channel number | - | X | - | - | X(1) | - | X(1) | - | - | - | - | - | (1) |
| 0x01 | Tune 2-Part Channel number | - | X | - | - | X(1) | - | X(1) | - | - | - | - | - | (1) |
| 0x88 | Global Direct (Frequency) Tune | - | X(1) | - | - | X(1) | - | X(1) | - | - | - | - | - | (3) |
| 0x89 | Global Direct (Tuner Channel) Tune | - | X(1) | - | - | X(1) | - | X(1) | - | - | - | - | - | (3) |
| 0x8A | Extended Global Direct (Frequency) Tune | - | X(1) | - | - | X(1) | - | X(1) | - | - | - | - | - | (3) |
|  | Input Select |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x02 | Select Composite A/V in | - | X(1) | - | - | X(1) | - | - | - | - | - | - | - | (1) |
| 0x03 | Select PC in | - | X(1) | - | - | X(1) | - | - | - | - | - | - | - | (1) |
| 0x04 | Select HDMI in | - | X(1) | - | - | X(1) | - | - | - | - | - | - | - | (1) |
| 0x05 | Select Component in | - | X(1) | - | - | X(1) | - | - | - | - | - | - | - | (1) |
| 0x06 | Select DVI in | - | X(1) | - | - | X(1) | - | - | - | - | - | - | - | (1) |
| 0x07 | Select DisplayPort in | - | X(1) | - | - | X(1) | - | - | - | - | - | - | - | (1) |
| 0x08 | Select USB in | - | O | O | - | O | - | - | - | - | - | - | - |  |
| 0x09 | Input Select (Generic) (2) | X | X(1) | - | - | X | X(1) | - | X(1) | - | - | - | - | (1)(2) |
| 0x0A-0x0F | Reserved Input | P | P | P | P | P | P | P | P | P | P | P | P |  |
|  | TV Setup |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x10 | Set Default Power On Source | - | X | - | - | O | - | - | - | - | - | - | O |  |
| 0x11-0x1F | Reserved Setup | P | P | P | P | P | P | P | P | P | P | P | P |  |
|  | Audio |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x20 | Set TV Speakers | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x21 | Set Digital Audio | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x22 | Set Analog Audio | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x23 | Set Default Power On Volume | - | X | - | - | O | - | - | - | - | - | - | O |  |
| 0x24 | Set Maximum Volume | - | X | - | - | O | - | - | - | - | - | - | O |  |
| 0x25 | Set Minimum Volume | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x26-0x3F | Reserved Audio | P | P | P | P | P | P | P | P | P | P | P | P |  |
|  | Display |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x40 | Set Blue Screen | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x41 | Set Brightness | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x42 | Set Color | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x43 | Set Contrast | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x44 | Set Sharpness | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x45 | Set Hue | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x46 | Set Backlight | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x47 | Set OSD Control | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x48 | Set Audio Only Display | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x49-0x5F | Reserved Display | P | P | P | P | P | P | P | P | P | P | P | P |  |
|  | Commercial |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x60 | Set Date | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x61 | Set Date Display Format | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x62 | Set Time | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x63 | Set Clock Brightness - Standby | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x64 | Set Clock Brightness - On | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x65 | LED Control | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x66 | Lock TV Power Button | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x67 | Lock TV Volume Buttons | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x68 | Lock TV Channel Buttons | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x69 | Lock TV Input Buttons | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x6A | Lock All Other TV Buttons | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x6B | Lock All TV Buttons INCLUDING Power | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x6C | Lock All TV Buttons EXCEPT Power | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x6D | Hotel Mode | - | O | - | - | O | - | - | - | - | - | - | - |  |
| 0x6E-0x7F | Reserved Commercial | P | P | P | P | P | P | P | P | P | P | P | P |  |
|  | Timers |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0x80 | Set Power Saving Profile | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x81 | Set Sleep Timer | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x82 | Set Wakeup Time | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x83 | Set Auto-Off Time | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x84 | Set Wakeup Source | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x85 | Set Initial Wakeup Volume | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x86 | Clear All Sleep/Wake Settings | - | O | - | - | O | - | - | - | - | - | - | O |  |
| 0x86 | Clear All Sleep/Wake Settings | - | O | - | - | O | - | - | - | - | - | - | O |  |

(1)   If the device supports the function, then support for the related CEC messaging is mandatory.  For example,

if a TV has a PC input, then it must support the CEC messaging to select the PC input.  But it is not mandatory

that the TV has a PC input.

(X without any note implies that support is mandated for both the Function AND the CEC Messaging.)

(2)  1st press brings up input select menu - subsequent presses advance to next item in list

(3) These tuning commands are mandatory only where direct tuning of DVB and/or ISDB programs is required

## HTNG Vendor Specific Remote Control Codes

The HTNG CEC specification defines additional remote control codes that are applicable for hospitality applications. The table below defines the HTNG Vendor Specific Remote Control Codes and which are mandatory and optional.

As for Section 4.4, devices that receive the raw remote control codes (TV or STB) must be able to translate the raw code to the corresponding HTNG Vendor Specific Remote Control Code and forward as necessary.

Devices that receive the HTNG Vendor Specific Remote Control Codes from the translator must provide functionality to implement the mandatory codes and should consider the benefit or otherwise of implementing optional codes.

Where a remote control must implement a specific button to be compliant with the HTNG CEC specification, then this too is identified.

| Legend |  |
| --- | --- |
| X | Mandatory |
| O | Optional |
| P | Prohibited |
| - | Don't Care |

| HTNG Vendor Specific Remote Control Pass-through Code | HTNG Vendor Specific Remote Control Pass-through Code Description | Mandatory or Optional Button | Remote Control Translator |  | Remote Control CEC Vendor Specific Message Receiver |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TV | STB Master | TV | STB Master | STB Slave | IAP | Optical Only Playback device | Multi-media Cloud Device * | Audio ** |  |
| 0x01 | Room Control GUI | O | X | X | - | O | O | - | - | - | - | If GUI not displaying, display the GUI for the in room controls. If GUI displaying, hide the GUI. |
| 0x02 | Closed Caption | X | X | X | X | X | X | - | O | X | - | If receiver is active device then toggle CC, else pass to active device. This feature must be available so it can be utilized where legally required |
| 0x03 | Closed Caption Language | O | X | X | O | O | O | - | - | - | - | Display available languages, User would then use navigation and select to choose desired language. |
| 0x04 | Sleep | O | X | X | O | O | - | - | - | - | - | Display the GUI to setup the sleep feature. STB needs to send CEC commands to other equipment |
| 0x05 | Wakeup | O | X | X | O | O | - | - | - | - | - | Display the GUI to setup the sleep feature. STB needs to send CEC commands to other equipment |
| 0x06 | Order/Buy | O | X | X | - | O | O | - | - | - | - | Single button on remote control to initiate a purchase. |
| 0x07 | Help | O | X | X | O | O | O | O | O | O | O | Display information relevant to the current feature. NOT intended for initiating the information services. |
| 0x08 | Information Services | O | X | X | - | O | O | - | - | - | - | Display GUI for information services: hotel, local, news, weather etc. |
| 0x09 | Navigate Back | X | X | X | X | X | X | X | X | X | X | Navigate to the previous screen/menu with each button press. |
| 0x0A | DVR Function | O | X | X | - | O | O | - | - | - | - | Display GUI to initiate recording of content - menu to allow choice of current or times future content. |
| 0x0B | # | O | X | X | O | O | O | O | - | - | - | Make # available to enable a telephone keypad |
| 0x0C | * | O | X | X | O | O | O | O | - | - | - | Make * available to enable a telephone keypad. |
| 0x0D | + | O | X | X | O | O | O | O | - | - | - | Make + available to enable a telephone keypad. |
| 0x0E | Language Select | O | X | X | O | O | O | - | - | - | - | Allow selection of users preferred language for system menus etc. |
| 0x0F | Live TV | O | X | X | - | O | O | - | - | - | - | Display the last broadcast TV channel - source could be from tuner or streamed. |
| 0x10 | ITV Menu | O | X | X | - | O | O | - | - | - | - | Display the interactive TV main menu. Mandatory for ITV solutions |
| 0x11 - 0x1F | Reserved for future use | P | P | P | P | P | P | P | P | P | P |  |
| 0x20 - 0x2F | Direct Input Select 1 - 16 | O | X | X | - | O | O | O | - | - | - |  |
| 0x30 - 0x3F | Direct Service Link 1 - 16 | O | X | X | - | O | O | - | - | - | - |  |
| 0x40 - 0x7F | Reserved / Future Use | P | P | P | P | P | P | P | P | P | P |  |
| 0x80 - 0xF9 | Reserved / Future Use | P | P | P | P | P | P | P | P | P | P |  |
| 0xFA - 0xFE | Vendor Reserved Code Set 1 - 5 | O | O | O | O | O | O | O | O | O | O |  |
| 0xFF | Keyboard | O | O | O | O | O | O | O | O | O | O |  |

# Appendices

## Glossary of Terms

For the purpose of this document the following terms have been defined as follows:

| Term | Definition |
| --- | --- |
| CEC | Consumer Electronics Control |
| FTG | “Free to Guest” |
| HDMI | High Definition Multimedia Interface |
| IAP | Intelligent Auxiliary Panel |
| SPDIF | Sony Philips Digital Interface |
| STB | Set Top Box |

## Implementation Notes

### Topologies Legend

## Referenced Documents

The table shows the documents upon which this document depends:

| Document Title | Location/URL |
| --- | --- |
| HDMI 1.3 | http://www.hdmi.org/manufacturer/specification.aspx |
| HDMI 1.4 | http://www.hdmi.org/manufacturer/specification.aspx (only HDMI members) |